Creality 3D K1 machine uses cura slicing software.

1. Download Slicing Software

Need to download cura5 slicing software and install it yourself:

Download link: GitHub - Ultimaker/Cura: 3D printer / slicing GUI built on top 	of the Uranium framework

2. Add printer

1. Entrance

2. Select K1 similar machine :Ender-3 S1

3. Renamed to Creality K1 or other name。

4. Just configure it to the K1 machine size, don't forget to start 		and end gcode synchronously。

Start G-code(To keep newlines):

M140 S0

M104 S0

START_PRINT EXTRUDER_TEMP=220 BED_TEMP=55

End G-code(To keep newlines):

END_PRINT

3. Import gcode configuration file.

- In the operation drop-down box on the right sidebar, click Manage Configuration Files:

2. Click the import button to upload the configuration file of the 	  	K1 machine we prepared in advance (tutorial attachment):

After the import is successful, we find the configuration file, select it, and click the fold button on the right:

3. Finally click to activate the configuration file to take effect:

K1 series machine configuration file:

cura_k1_pla.curaprofile

4. Save to desktop after slicing

After all configuration operations are completed, we import the model, select the imported configuration file, and click the slice button:

- After slicing is successful, we save the gcode file to the desktop/disk space。

© 2014-2023 Shenzhen Chuangxiang 3D Technology Co., Ltd. All rights reserved.

Feedback email: kefu@creality.com